# Kisan CattleVision: Comprehensive User Manual & Operations Guide

## CHAPTER 1: SYSTEM OVERVIEW & ARCHITECTURE

### 1.1 Purpose and Key Capabilities

The Kisan CattleVision system is an end-to-end, edge-native computer vision and livestock intelligence platform. Designed specifically for offline deployment on Nvidia Jetson embedded hardware (Jetson Orin Nano, Xavier NX), it continuously analyzes multiple live RTSP camera streams to automate livestock behavior monitoring, cattle identification, and farm worker attendance tracking without requiring wearable sensors or continuous internet access.

The system encompasses three primary functional capabilities:

Cattle Behavior & Health Monitoring: Real-time localization of cattle, posture classification (Standing vs. Lying), feeding activity detection (Feeding vs. Idle), and anomaly risk evaluation.

Automated Ear Tag Identification (OCR): High-precision localization of plastic ear tags, character segmentation, and optical character recognition (EasyOCR) coupled with fuzzy dictionary correction against a master herd registry.

Worker & Visitor Re-Identification (Re-ID): ArcFace-powered facial verification, 6-of-10 consensus voting, priority known matching, automatic visitor template registration (UNK_XXX), and clothing color histogram recovery.

### 1.2 High-Level System Architecture

### 1.3 Subsystem Directory Breakdown

The repository contains three primary application packages:

| Directory / Package | Purpose & Role | Key Entry Scripts |
| --- | --- | --- |
| Kisan_Jetson/ | Core edge computing engine. Ingests RTSP streams, runs YOLOv8 and FaceNet models, classifies behaviors, and hosts the local FastAPI dashboard. | jetson_app.py, jetson_pipeline.py, Genz_person_reid.py |
| 4_jetson_connection_member_monitoring/ | Cloud sync daemon. Monitors local CSV attendance/behavior logs and synchronizes them with AWS S3 buckets. Downloads remote member registrations. | s3_sync_client.py |
| Kisan_Jetson_Mobile/ | Remote companion suite. Contains the stream forwarder daemon, EC2 video relay server, and cross-platform Flutter application. | forwarder.py, app.py, flutter_app/ |
| Documents/ | Technical documentation, user guides, architectures, and compile scripts. | USER_MANUAL.md, README.md, ARCHITECTURE.md |

### 1.4 End-to-End Processing Workflow

## CHAPTER 2: HARDWARE SETUP & PHYSICAL DEPLOYMENT

### 2.1 Hardware Bill of Materials (BOM)

| Component | Recommended Specification | Purpose |
| --- | --- | --- |
| Edge Compute Unit | Nvidia Jetson Orin Nano Developer Kit (8GB RAM, NVMe SSD) | Runs AI models, OCR, and hosting server. |
| IP Cameras (Feeding Lane) | 1080p Full HD Bullet IP Camera, 4mm/6mm lens, 30 FPS RTSP | Captures clear top-down views of cattle ear tags. |
| IP Camera (Entrance Gate) | 1080p Full HD Dome/Bullet IP Camera, 2.8mm wide-angle lens | Captures frontal facial views of entering personnel. |
| Network Switch | 8-Port Gigabit PoE (Power over Ethernet) Switch (802.3af/at) | Powers IP cameras and routes video data to Jetson. |
| Enclosure | Weatherproof IP66 Wall-Mount Cabinet | Protects Jetson from farm dust, ammonia, and moisture. |
| Power Supply | 19V / 45W DC Power Adapter with Surge Protection | Ensures stable power delivery during peak GPU loads. |
| Cabling | Outdoor-rated Shielded Cat6 Ethernet Cable | Prevents electromagnetic interference and weather damage. |

### 2.2 Physical Installation & Camera Mounting Guidelines

Proper camera positioning is critical for maximizing OCR accuracy and face recognition consistency:

1. Feeding Lane Camera Placement:

Height: Mount the camera at 2.5 to 2.8 meters directly above the feeding lane barrier.

Angle: Tilt the camera downward at a 35° to 45° angle facing the cattle stalls.

Rationale: This angle provides an unobstructed top-down perspective on yellow ear tags, preventing neighboring cattle from blocking the view while cows eat.

2. Entrance Gate Camera Placement:

Height: Mount the camera at 1.8 meters directly facing the entry doorway.

Angle: Angle downward slightly at 5° to 10°.

Rationale: Captures straight frontal face angles, ensuring high cosine similarity scores (> 0.80) during face recognition.

3. Illumination Requirements:

Maintain minimum ambient lighting of 50 Lux in feeding and entry zones.

Avoid positioning cameras directly opposite bright eastern/western windows to prevent intense backlighting and silhouetting.

### 2.3 Local Network Topology & IP Configuration

The Jetson server and IP cameras must reside on an isolated static subnet to eliminate routing drops and stream latency:

## CHAPTER 3: SOFTWARE INSTALLATION & PREREQUISITES

### 3.1 Operating Environment Requirements

Host OS: Ubuntu 22.04 LTS (installed via Nvidia JetPack 6.0 / 6.1).

CUDA Toolkit: Version 12.2 or higher.

Python Version: Python 3.10 or Python 3.11.

TensorRT: Pre-installed via JetPack.

### 3.2 Installation Commands

1. Navigate to the Project Root:

`bash

cd "F:/Kisan Pro Downlods Git/Cattle behaviour Analysis/Cattle Behaviour Monitoring/Cattle Behaviour analysis Jetson version"

`

2. Set Up a Python Virtual Environment (Recommended):

`bash

python3 -m venv venv

source venv/bin/activate

`

3. Install All Required Dependencies:

`bash

pip install --upgrade pip

pip install -r requirements.txt

`

### 3.3 Core Configuration File (`Kisan_Jetson/config.json`)

The behavior of the edge server is governed by config.json:

{
  "user_name": "Farmer Geetha",
  "phone": "9876543210",
  "farm_name": "Kisan Gitam Farm",
  "camera_feeds": {
    "cam1": "rtsp://admin:admin123@192.168.0.162:554/stream1",
    "cam2": "rtsp://admin:admin123@192.168.0.112:554/stream1",
    "cam3": "rtsp://admin:admin123@192.168.0.110:554/stream1"
  },
  "enable_tensorrt": false,
  "confidence_threshold": 0.50,
  "reid_threshold": 0.58,
  "stranger_threshold": 0.75
}

## CHAPTER 4: OPERATING THE EDGE AI SERVER (`Kisan_Jetson`)

### 4.1 Launching the Live Server

To start the live video processing server and web dashboard:

1. Open a terminal and navigate to Kisan_Jetson:

`bash

cd Kisan_Jetson

python jetson_app.py

`

2. Look for the startup log confirmation:

`

[OK] Loaded 6 known face embeddings.

[OK] Loaded master cattle list. Total: 4

[OK] ChromaDB Vector DB ready.

[OK] EasyOCR reader ready.

[Kisan CattleVision] Starting on http://0.0.0.0:8000

`

3. Open any web browser and visit: http://localhost:8000

### 4.2 Web Dashboard Interface Overview

The web dashboard provides a complete real-time operating center:

Live Video Streaming: Multi-grid view of all active camera feeds with colored bounding box overlays (Blue for cattle, Green for verified members, Orange for unknown visitors).

Herd Activity Feed: Live table displaying individual cattle IDs, active postures (Standing/Lying), and cumulative daily feeding duration.

Attendance Monitor: Timestamped record of farm workers verified today with audio greeting indicators.

### 4.3 Livestock Tag OCR & Master List Verification

1. Ear Tag Detection: The custom YOLOv8 model detects yellow ear tag regions on cattle heads.

2. EasyOCR Extraction: The detected crop is processed via EasyOCR to read alphanumeric characters (e.g., A145, B643).

3. Master List Correction: The extracted string is matched against storage/master_cattle_list.txt. If minor character swaps occur due to dirt or angle (e.g., OCR reads P643), the fuzzy distance algorithm corrects it to the master list entry (B643).

### 4.4 Person Re-ID: Consensus Voting & Priority Margins

1. Face Detection & Embeddings: Faces are cropped via yolov8n-face and converted to 512-D vectors via InceptionResnetV1.

2. Priority Matching Policy: If cosine similarity with a registered member is k_sim >= 0.58 with a 0.10 separation margin, the system prioritizes the known member identity, avoiding false unknown visitor registrations.

3. Consensus Voting: A track requires 6 matching votes out of 10 sequential frames before identity locks permanently.

4. Clothing Color Histogram Recovery: When an individual is temporarily occluded behind a pillar, their 3D HSV color histogram matches new tracks upon re-entry, restoring the continuous track ID.

## CHAPTER 5: CLOUD SYNCHRONIZATION (`4_jetson_connection_member_monitoring`)

### 5.1 Cloud Synchronization Architecture

The S3 sync client runs as a background service to push edge logs to AWS and pull new worker templates registered via the mobile app.

### 5.2 Launching the S3 Sync Daemon

1. Navigate to the connection directory:

`bash

cd "4_jetson_connection_member_monitoring"

`

2. Verify AWS credentials in .env:

`ini

AWS_ACCESS_KEY_ID=your_aws_access_key

AWS_SECRET_ACCESS_KEY=your_aws_secret_key

S3_BUCKET_NAME=your_farm_s3_bucket

FARM_ID=Farmer_Geetha_9876543210_Kisan_Gitam_Farm

`

3. Start the sync script:

`bash

python s3_sync_client.py

`

## CHAPTER 6: MOBILE COMPANION APP (`Kisan_Jetson_Mobile`)

### 6.1 Stream Forwarder Setup (`stream_forwarder/`)

To stream live camera feeds from the edge device to the mobile app across the internet:

1. Navigate to Kisan_Jetson_Mobile/stream_forwarder:

`bash

cd Kisan_Jetson_Mobile/stream_forwarder

`

2. Configure forwarder_config.json with your EC2 relay URL:

`json

{

"ec2_url": "http://your-ec2-public-ip:8080",

"forward_cameras": ["cam1", "cam2", "cam3"],

"forward_interval_sec": 0.1

}

`

3. Start the forwarder daemon:

`bash

python forwarder.py

`

### 6.2 Running the Flutter Mobile App (`flutter_app/`)

1. Ensure Flutter SDK is installed on your workstation.

2. Build and launch the mobile app:

`bash

cd Kisan_Jetson_Mobile/flutter_app

flutter run

`

## CHAPTER 7: STANDARD OPERATING PROCEDURES (SOPs)

### SOP 1: Registering a New Farm Worker / Member

1. Open the web dashboard at http://localhost:8000.

2. Click Register Member in the navigation menu.

3. Enter the worker's Full Name and Role (e.g., "Supervisor", "Feeder").

4. Upload 3 to 5 clear frontal face photos (or capture directly using the camera tool).

5. Click Submit & Compile Embeddings. The system will generate ArcFace vectors and update known_embeddings.pkl without requiring a server restart.

### SOP 2: Updating the Master Cattle Herd List

1. Open Kisan_Jetson/storage/master_cattle_list.txt in any text editor.

2. Add, remove, or modify tag IDs (one identifier per line, e.g., A145, B643).

3. Save the file. The pipeline automatically re-reads the master list on the next detection cycle.

### SOP 3: Resetting Stranger History (Starting from Scratch)

If you wish to clear all dynamically registered unknown visitor profiles:

1. Stop the application server (Ctrl + C in the server terminal).

2. Execute the database reset command:

`bash

python -c "import os, pickle; [os.remove(os.path.join(d, f)) for d in [r'Kisan_Jetson/static/unknown_faces'] if os.path.exists(d) for f in os.listdir(d) if os.path.isfile(os.path.join(d, f))]; pickle.dump({'people': {}}, open(r'Kisan_Jetson/embeddings/unknown_embeddings.pkl', 'wb')); print('Wiped stranger database successfully.')"

`

3. Restart jetson_app.py.

## CHAPTER 8: TROUBLESHOOTING & MAINTENANCE

### 8.1 Troubleshooting Decision Flowchart

### 8.2 Troubleshooting Matrix

| Symptom | Probable Cause | Verification & Fix Action |
| --- | --- | --- |
| Black screen or RTSP timeout error | Camera IP collision or disconnected PoE line. | Ping camera IP (ping 192.168.0.162). Verify PoE switch port LED is solid green. |
| Ear tag OCR misreads characters | Motion blur or camera angle too shallow. | Ensure camera is tilted at 35°-45°. Verify tag is listed in master_cattle_list.txt. |
| Known member recognized as Unknown (UNK_XXX) | Steep facial tilt or registration photo missing. | Ensure registered worker has clean frontal photos in known_embeddings.pkl. Priority threshold is set to 0.58. |
| Low FPS / Video latency on Jetson | Thread contention or CPU-bound processing. | Verify cv2.setNumThreads(4) is configured in jetson_pipeline.py. Enable TensorRT engines if available. |
| S3 Sync client authentication failure | Expired or incorrect AWS credentials. | Verify AWS_ACCESS_KEY_ID and AWS_SECRET_ACCESS_KEY in 4_jetson_connection_member_monitoring/.env. |

## CHAPTER 9: SYSTEM VERIFICATION & SANITY CHECKLIST

[x] All 3 RTSP camera streams reachable via local static subnet (192.168.0.0/24).

[x] YOLOv8 cattle detection bounding boxes render with confidence > 0.50.

[x] Ear tag OCR correctly locks registered IDs (A145, B643, D675, A675).

[x] Behavior logs written to storage/behavior_logs.csv every 60 seconds.

[x] Known member facial verification completes with audio greeting trigger.

[x] Dynamic stranger registration creates isolated UNK_XXX entries.

[x] S3 cloud sync pushes attendance logs every 5 minutes.