User Manual & Step-by-Step Reimplementation Guide

Cattle Ear Tag Detection & Recognition System

Welcome & Document Purpose

This manual provides an exhaustive, step-by-step operational guide for machine learning engineers, farm automation specialists, and software developers to re-implement, train, deploy, and integrate the Cattle Ear Tag Detection and Optical Character Recognition (OCR) system from scratch.

SECTION 1: ENVIRONMENT SETUP & PREREQUISITES

1.1 Hardware Prerequisites

Computer: Windows 10/11, Ubuntu 20.04/22.04 LTS, or macOS (Apple Silicon).

GPU (Recommended for Real-Time Speed): NVIDIA GPU with CUDA compute capability >= 6.0 (e.g. GTX 1660, RTX 3060/4090, Jetson Orin Nano).

RAM: Minimum 8 GB (16 GB recommended for large dataset training).

Storage: 10 GB free disk space.

1.2 Software Prerequisites

Python: Version 3.9, 3.10, or 3.11.

NVIDIA CUDA Toolkit: Version 11.8 or 12.1 (if utilizing GPU).

SECTION 2: STEP-BY-STEP REIMPLEMENTATION GUIDE

Step 1: Clone Repository & Create Virtual Environment

Open your terminal (PowerShell on Windows, Bash on Linux/macOS):

# Navigate to desired directory
cd "F:\Kisan Pro Downlods Git\Cattle behaviour Analysis\Cattle Behaviour Monitoring\Cattle Ear Tag Training Code"

# Create a clean virtual environment named 'venv'
python -m venv venv

# Activate the virtual environment:
# On Windows (PowerShell):
venv\Scripts\Activate.ps1
# On Windows (Command Prompt):
# venv\Scripts\activate.bat
# On Linux/macOS:
# source venv/bin/activate

Step 2: Install Required Dependencies

Upgrade pip and install all pinned dependencies:

python -m pip install --upgrade pip
pip install -r requirements.txt

Step 3: Verify GPU & Submodule Imports

Run this one-line sanity test in your terminal:

python -c "import torch, cv2, easyocr, ultralytics; from src.pipeline.ear_tag_pipeline import EarTagPipeline; print('CUDA Available:', torch.cuda.is_available()); print('ALL ENGINES READY!')"

Expected Output:

CUDA Available: True (or False if on CPU)
ALL ENGINES READY!

SECTION 3: DATASET PREPARATION & ANNOTATION

3.1 Dataset Structure & YOLO Annotation Format

Cattle ear tag detection uses standard YOLO format:

For every image file (e.g. cow100.jpg), there must be a corresponding text label file (e.g. cow100.txt).

Each row in the .txt file specifies: <class_id> <x_center> <y_center> <width> <height> (normalized between 0.0 and 1.0).

Since we are detecting ear tags, class_id is always 0.

Example cow100.txt:

0 0.428512 0.354129 0.124500 0.186200

3.2 Splitting Raw Datasets

To split a raw collection of images and labels into 70% Train, 20% Validation, and 10% Test:

python scripts/split_data.py \
  --images "path/to/raw_images" \
  --labels "path/to/raw_labels" \
  --output dataset \
  --train-ratio 0.7 \
  --val-ratio 0.2 \
  --test-ratio 0.1

SECTION 4: HOW TO TRAIN / FINE-TUNE THE YOLO MODEL

4.1 Training Using CLI

To train the YOLOv8 ear tag detection model on your prepared dataset:

# Standard 100-Epoch Training on GPU
python scripts/train.py \
  --model yolov8n.pt \
  --data config/data.yaml \
  --epochs 100 \
  --batch-size 16 \
  --imgsz 640 \
  --name cattle_ear_tag_run

> IMPORTANT Windows Note: On Windows operating systems, DataLoader multiprocessing can deadlock if workers > 0. The trainer automatically defaults to workers=0 for safe execution.

4.2 Where Weights Are Saved

When training completes, the best weights are automatically saved to models/best.pt.

Complete epoch training plots (loss curves, mAP charts, confusion matrices) are saved in runs/detect/cattle_ear_tag_run/.

SECTION 5: HOW TO RUN DETECTION & OCR INFERENCE

5.1 Run Full End-to-End Pipeline (Detect + OCR)

To process a folder of cattle images and extract both bounding boxes and read tag numbers:

# Process sample test set
python scripts/run_ocr_pipeline.py --input dataset/algo_test --output output/results

What this outputs:

Annotated Images: Saved in output/results/annotated_*.jpg with bounding boxes and tag ID badges.

CSV Spreadsheet Report: Saved in output/results/ear_tag_results.csv.

JSON Metadata File: Saved in output/results/ear_tag_results.json.

5.2 Process a Single Image & Save Tag Crops

python scripts/run_ocr_pipeline.py \
  --input dataset/algo_test/tc1.jpg \
  --save-crops \
  --output output/single_test

5.3 Run Detection-Only (Without OCR)

If you only need bounding boxes without running text recognition:

python scripts/run_inference.py --input dataset/algo_test --output testing_results/predict

SECTION 6: PYTHON API INTEGRATION IN CUSTOM APPLICATIONS

You can easily integrate the pipeline into your own Python applications, Flask/FastAPI backends, or video processing loops:

import cv2
from src.pipeline.ear_tag_pipeline import EarTagPipeline

# 1. Initialize the pipeline
pipeline = EarTagPipeline(
    model_path="models/best.pt",
    det_conf=0.35,       # Detection confidence threshold
    ocr_conf=0.30,       # OCR confidence threshold
    device="auto"        # Uses GPU if available
)

# 2. Process an image file or OpenCV frame
image_path = "dataset/algo_test/tc1.jpg"
result = pipeline.process_image(
    image_input=image_path,
    save_annotated_to="output/annotated_tc1.jpg"
)

# 3. Access structured detection & OCR records
for record in result.records:
    print(f"Detected Tag ID : {record.tag_id}")
    print(f"YOLO Confidence : {record.det_confidence:.2f}")
    print(f"OCR Confidence  : {record.ocr_confidence:.2f}")
    print(f"Bounding Box    : [{record.box_x1}, {record.box_y1}, {record.box_x2}, {record.box_y2}]")
    print(f"Timestamp       : {record.timestamp}")

SECTION 7: HYPERPARAMETER TUNING & CONFIGURATION

All system parameters can be adjusted directly in config/config.yaml:

detection:
  img_size: 640          # 640 is standard; use 1280 for distant / small tags
  conf_threshold: 0.35   # Lower to 0.25 to detect more tags; raise to 0.50 to reduce false boxes
  iou_threshold: 0.50    # Non-Maximum Suppression overlap threshold
  device: "auto"         # 'auto', '0', 'cpu'

ocr:
  languages: ["en"]      # Recognition language
  use_gpu: true          # Use GPU acceleration for EasyOCR
  min_text_length: 2     # Minimum characters to consider a valid tag
  conf_threshold: 0.30   # Minimum OCR confidence required
  preprocess:
    resize_factor: 2.0   # Scale small crops up 2x for sharper character strokes
    apply_clahe: true    # Enable CLAHE contrast enhancement (essential for dusty tags)
    denoise: true        # Enable bilateral noise reduction

SECTION 8: HARDWARE & PHYSICAL CAMERA CALIBRATION

Camera Angle: Position cameras at a 25° to 35° downward angle toward cattle head level.

Mounting Height: Mount cameras 2.2 to 2.8 meters above the ground.

Lighting: Avoid placing bright lights directly behind cattle (backlighting causes dark silhouette tags). Place diffused 5000K LED wash lights above the camera facing the animals.

Resolution: Ensure the camera provides at least 1080p FHD. Cattle ear tags must be at least 64x64 pixels within the frame.

SECTION 9: TROUBLESHOOTING & FAQ

Q1: PyTorch hangs or freezes during training on Windows

Solution: Ensure workers=0 in scripts/train.py. Windows multiprocessing spawn mode can cause deadlock if workers > 0.

Q2: CUDA Out of Memory (OOM) during training

Solution: Reduce batch size using --batch-size 8 or --batch-size 4.

Q3: Tag is detected but the tag ID is blank (ear_tag)

Cause: OCR confidence was below threshold (< 0.30) or tag was too dirty/blurry.

Solution: Enable --save-crops to inspect the cropped tag image in output/results/crops/. Lower --ocr-conf 0.20 or ensure apply_clahe: true in config/config.yaml.

Q4: Moving the folder to a new computer causes path errors

Solution: The code uses dynamic relative path resolution via src/config_loader.py. Simply run the commands from the root directory or pass paths relative to the project root.