🚀 KisanPro Master Commands & Execution Guide

This document contains the complete set of commands to launch, run, and operate all components of the KisanPro Integrated Cattle Health & Monitoring System.

📋 System Architecture Overview

The system consists of 4 decoupled subsystems running across cloud, local GPU, WSL2 Linux, and Android:

┌────────────────────────────────────────────────────────┐
│               1. AWS EC2 Cloud Gateway                 │
│              (Host: 35.153.224.84:5000)                │
└──────────────────────────┬─────────────────────────────┘
                           │
       ┌───────────────────┴───────────────────┐
       ▼                                       ▼
┌──────────────────────────────┐    ┌──────────────────────────────┐
│  2. Local PC GPU Worker      │    │  4. Android Mobile Client    │
│  (Windows PowerShell)        │    │  (Flutter Unified APK)       │
│  - Keypoint Detection (CUDA) │    │  - Milk, Vaccine, Weight 3D  │
│  - Ramanujan Body Geometry   │    └──────────────────────────────┘
└──────────────┬───────────────┘
               │ (Local HTTP 9090)
               ▼
┌──────────────────────────────┐
│  3. WSL2 Linux 3D Server     │
│  - TRELLIS.2 4B Foundation   │
│  - BiRefNet Segmentation     │
│  - GLB Mesh Generator        │
└──────────────────────────────┘

🪟 1. Terminal 1: WSL 3D AI Reconstruction Server (Local PC)

Environment: WSL2 Ubuntu Linux

Role: Generates interactive 3D .glb meshes using TRELLIS.2 4B generative foundation model on port 9090.

Step-by-Step Commands:

# 1. Open Windows PowerShell and enter WSL2
wsl

# 2. Navigate to the 3D Server directory
cd /mnt/f/JRF/Combined_Weight_Monitoring/wsl_3d_server

# 3. Activate the conda environment
source /home/thipp/miniconda3/bin/activate trellis2

# 4. Start the 3D Server
python app.py

Or as a Single Copy-Paste Line in WSL:

cd /mnt/f/JRF/Combined_Weight_Monitoring/wsl_3d_server && source /home/thipp/miniconda3/bin/activate trellis2 && python app.py

> Expected Terminal Output:

> Cattle 3D Backend starting on port 9090...

> Running on http://127.0.0.1:9090 (Leave this window open)

🪟 2. Terminal 2: Local GPU Inference Worker (Windows PowerShell)

Environment: Native Windows PowerShell (Outside WSL)

Role: Continuously polls pending tasks from AWS EC2, runs CUDA keypoint detection, calculates physical dimensions (OBL, WH, HG, HL), executes Extra Trees body weight regression, calls WSL 3D engine, and uploads results back to cloud.

Step-by-Step Commands:

# 1. Open a new Windows PowerShell window
# 2. Navigate to the GPU worker folder
cd F:\JRF\Combined_Weight_Monitoring\local_pc_worker

# 3. Start the GPU Worker daemon
python gpu_worker.py

> Expected Terminal Output:

> [INFO] Loading models on cuda...

> [OK] Loaded side KP model

> [OK] Loaded back KP model

> Kisan Pro Local GPU Worker Daemon Active

> Target Cloud: http://35.153.224.84:5000 (Leave this window open)

🪟 3. Terminal 3: KisanPro Unified Mobile Application (Flutter Android)

Environment: Native Windows PowerShell

Role: Compiles and installs the Flutter mobile app on connected Android phone (Realme RMX5061 / TOOJ6HXOW4NBKVS8).

Step-by-Step Commands:

# 1. Open a new Windows PowerShell window
# 2. Navigate to the Unified App folder
cd F:\JRF\kisanpro_unified_app

# 3. Verify phone connection via ADB
& "$env:LOCALAPPDATA\Android\Sdk\platform-tools\adb.exe" devices

# 4. Build Debug APK
flutter build apk --debug

# 5. Install APK directly onto connected phone
& "$env:LOCALAPPDATA\Android\Sdk\platform-tools\adb.exe" -s TOOJ6HXOW4NBKVS8 install -r build\app\outputs\flutter-apk\app-debug.apk

# 6. Launch the App on phone
& "$env:LOCALAPPDATA\Android\Sdk\platform-tools\adb.exe" -s TOOJ6HXOW4NBKVS8 shell monkey -p com.kisanpro.unified 1

Pre-Built Standalone APK Install Commands:

# Directly install pre-compiled debug APK from repository
& "$env:LOCALAPPDATA\Android\Sdk\platform-tools\adb.exe" install -r "F:\JRF\Combined_Weight_Monitoring\apk\app-debug.apk"

☁️ 4. Terminal 4: AWS EC2 Cloud Gateway (Remote Server)

Host IP: 35.153.224.84

Port: 5000

SSH Key: C:\Users\GITAM\Downloads\cattle-gateway-key.pem

Role: REST API endpoints for user authentication, cattle profile synchronization, job queue dispatching, and AWS S3 photo/mesh storage.

Step-by-Step Commands (from Windows PowerShell to EC2):

# 1. SSH into EC2 Server
ssh -i C:\Users\GITAM\Downloads\cattle-gateway-key.pem ec2-user@35.153.224.84

Commands on Remote EC2 Linux Server:

# 2. Navigate to Cloud Gateway Directory
cd /home/ec2-user/cloud_gateway

# 3. Activate Python virtual environment
source venv/bin/activate

# 4. Stop any existing gateway instances
pkill -f gunicorn

# 5. Start Production Daemon
gunicorn -w 2 -b 0.0.0.0:5000 app_cloud:app --daemon

# 6. Verify Service Health
curl http://localhost:5000/api/status

> Expected JSON Output:

> {"device":"cloud-gateway","message":"Cloud Gateway Active","ready":true,"weight_models_loaded":1}

📊 Complete Health Verification Matrix

| Subsystem | Execution Location | Port / Protocol | Health Check Command | Expected Result |
| --- | --- | --- | --- | --- |
| AWS Cloud Gateway | AWS EC2 Linux | Port 5000 (HTTP) | curl http://35.153.224.84:5000/api/status | {"ready": true, "message": "Cloud Gateway Active"} |
| 3D Reconstruction Engine | Local PC (WSL2) | Port 9090 (HTTP) | curl http://localhost:9090/health | {"status": "healthy", "model": "trellis2"} |
| Local GPU Worker | Local PC (PowerShell) | CUDA / PyTorch | Monitor Terminal Logs | [Worker] Polling for tasks... |
| Unified Mobile App | Android Phone | USB ADB / Flutter | adb devices | TOOJ6HXOW4NBKVS8 device |

🔄 Daily Quickstart Workflow (Start Everything in 3 Steps)

Start WSL 3D Server:

Open PowerShell -> wsl -> cd /mnt/f/JRF/Combined_Weight_Monitoring/wsl_3d_server && source /home/thipp/miniconda3/bin/activate trellis2 && python app.py

Start GPU Worker:

Open PowerShell -> cd F:\JRF\Combined_Weight_Monitoring\local_pc_worker && python gpu_worker.py

Open KisanPro App on Phone:

Navigate to Weight & 3D tab -> Capture or upload cattle photos -> Tap ESTIMATE WEIGHT & 3D MODEL.